MATERIAL SAFETY DATA SHEET

Date Prepared: 7-1-11

SECTION 1 – PRODUCT INFORMATION

Distributor:    Swiss Clean, Inc.

Address:  3650 – 44th St. SE, Kentwood, MI  49512    			Emergency Phone:  1-800-424-9300

Trade Name: Phoenix Burlesque						Product Type:   Floor Stripper

## SECTION 2 – INGREDIENTS

| Chemical Name/Common Name | CAS No. | % (Optional) | TLV/PEL | TITLE III, SECT. 313 |
| --- | --- | --- | --- | --- |
| 2-Amino Ethanol | 141-43-5 | PROP. | 3 ppm | NO |
| Butoxyethoxy Propanol | 124-16-3 | PROP. | Not Established | NO |
| 2-Butoxyethanol | 111-76-2 | PROP. | 25 ppm | YES |
| Sodium Hydroxide | 1310-73-2 | PROP | 2 mg/m3 | NO |

## SECTION 3 – PHYSICAL DATA

## Flash Point (°F):   None to boiling (TOC)

Flammable Limits:   N\A

Unusual Fire & Explosion Hazards: Heated material may liberate toxic fumes. Wear self contained breathing apparatus.

# SECTION 4 – FIRE AND EXPLOSION HAZARD DATA

# Flash Point (°F):   None to boiling (TOC)

Flammable Limits:   N\A

Unusual Fire & Explosion Hazards: Heated material may liberate toxic fumes. Wear self contained breathing apparatus.

## SECTION 5 – REACTIVITY DATA

## Stability:  Stable

Conditions to Avoid:   None

Incompatibility: Acids and Oxidizers

Hazardous Decomposition Products: Carbon Oxides and Amines

Hazardous Polymerization: Will not occur.

## SECTION 6 – HEALTH HAZARD DATA

## Route(s) of Entry:    Inhalation: No   Skin:   Yes   Ingestion: Yes	Threshold Limit Value:   Not Est.

Effects of Overexposure: Causes skin and eye irritation. Harmful if swallowed. May cause severe mucosal damage and gastric distress. May cause systemic intoxication.

## SECTION 7 – EMERGENCY AND FIRST AID PROCEDURES

Eyes: Rinse thoroughly with water for at least fifteen minutes. If irritation persists seek medical attention.

Skin: Thoroughly wash with water and apply emollient creme. If irritation persists seek medical attention.

Ingestion: Do not induce vomiting.  Give milk, egg white, gelatin, or if none of these are available give large quantities of water.  Seek immediate medical attention. Never give anything by mouth to an unconscious person.

Conditions Aggravated by Exposure: May aggravate existing dermatitis and irritate sensitive skin. May irritate any pre-existing pulmonary conditions.

Listed Carcinogen, Mutagen, Teratogen (Ingredient, Source, Amount):   None

## SECTION 8 – SPECIAL PROTECTION INFORMATION

## Respiratory Protection: Generally not required

Ventilation:   Local

Protective Gloves: Rubber or latex

Eye Protection: Goggles or safety glasses with side shields

Other Protective Equipment: Rubber boots should be worn if working in standing solutions.

## SECTION 9 – SPILL OR LEAK PROCEDURES

## Steps to be taken If material is released or spilled: Recontainerize by mopping, wet vacuum, or using a suitable absorbent. Rinse remaining material to drains. Be cautious of slippery floors.

Waste Disposal Method: Follow local, state, and federal regulations.

## SECTION 10 – STORAGE AND HANDLING INFORMATION

## Precautions to be taken in handling and storage: Store between 40°F and 120°F.

Other Precautions: Remove contaminated clothing. Wash all contaminated clothing thoroughly before reuse. Keep out of reach of children. For professional use only.

The information contained in this MSDS was obtained from current and reliable sources, however the data is provided without any warranty, expressed or implied, regarding its correctness or accuracy. Since the conditions or handling, storage and disposal of this product are beyond the control of this company, it is not responsible for loss, injury and expense arising out of the product's improper use. No warranty, expressed or inferred, regarding the product described in this MSDS shall be created or inferred by any statement in this MSDS. Various government agencies may have specific regulations regarding the transportation, handling, storage, use or disposal of this product which may be covered by this MSDS. The user is responsible for fuB compliance.