MATERIAL SAFETY DATA SHEET

Date Prepared: 7-1-11

SECTION 1 – PRODUCT INFORMATION

Distributor:    Swiss Clean, Inc.      	                			Emergency Phone:  1-800-424-9300

Address:        3650 – 44th St. SE, Kentwood, Mi  49512

Trade Name:  Phoenix Precoat                                                                  Product Type:  All Purpose Cleaner

## SECTION 2 – INGREDIENTS

| Chemical Name/Common Name | CAS No. | % (Optional) | TLV (Source) |
| --- | --- | --- | --- |
| Lauryl Dimethyl Amine Oxide | 1643-20-5 | 1 – 5% | None Established |
| Nonionic Surfactant | 9016-45-9 | 1 – 5% | None Established |
| Tetra sodium EDTA | 64-02-8 | 1 – 5% | None Established |
| Water | 7732-18-5 |  | None Established |

## SECTION 3 – PHYSICAL DATA

Boiling Point (°F):  212°              Specific Gravity (H2O=1.0):  1.0 ± 0.005                          pH:  12.5 ± 0.5

Vapor Pressure (mm Hg):  Not Determined                                   Vapor Density (Air=1):  Not Determined

Solubility in water:  Complete          Evaporation Rate (vs. H2O):   About the Same

Appearance and Odor: Clear Green liquid with lemon odor

## SECTION 4 – FIRE AND EXPLOSION HAZARD DATA

Flash Point (T.C.C.): None to Boiling          Flammable Limits:  N/A

Extinguishing Method:  Product is non-combustible.  Water Spray (Fog), Dry Chemical, CO2 or Foam may be used where

product is stored.

Special Firefighting Procedures:  Do not enter confined fire spaces without protective clothing and self-contained air supply.    Unusual Fire and Explosion Hazards:  None

## SECTION 5 – REACTIVITY DATA

Stability:  Stable Incompatibility:  Strong acids/oxidizers.  Do not mix with chlorinated detergents (bleach).

Hazardous Decomposition Products:  Burning may produce oxides of carbon and other substances.

## SECTION 6 – HEALTH HAZARD DATA

Primary Routes of Exposure:  Eye, Skin, Oral

Signs and Symptoms of Overexposure (Acute):  Direct contact with eyes may cause mild to moderate irritation with local redness & swelling of conjunctiva.  Prolonged or repeated skin contact may cause irritation.  Ingestion may cause irritation of mouth, throat & stomach with possible nausea and vomiting.

Signs and Symptoms of Overexposure (Chronic):  None currently known.

Medical Conditions Aggravated by Overexposure:  An existing dermatitis.

Carcinogen or Suspect Carcinogen Ingredients:  None

## SECTION 7 – EMERGENCY AND FIRST AID PROCEDURES

Eyes:  Flush with water for at least 15 minutes, call a physician immediately.

Skin:  Wash affected area with large amounts of soap & water.  If irritation persists, call a physician.

Ingestion:  DO NOT induct vomiting.  Dilute by drinking water.  Call a physician immediately.

Inhalation:  Not an expected route of exposure.  If  irritation, remove subject to fresh air.

## SECTION 8 – SPECIAL PROTECTION INFORMATION

Respiratory Protection:  Not required under normal use conditions with good general ventilation.  Protect against generated mists/sprayback.

Ventilation Requirements:  Normal    Eye Protection:  Safety Glasses/Goggles

Protective Gloves:  Rubber/PVC     Other Protective Clothing:  Appropriate to minimize contact during use.

## SECTION 9 – SPILL OR LEAK PROCEDURES

Steps to be Taken if Released or Spilled:  Do not touch or walk through.  Stop leak if you can without risk.  Dike ahead of large

spills to prevent run-off.  Mop, pump or take up with sand or other inert absorbent and reclaim into containers for reuse, recycle or proper disposal.

Waste Disposal Methods:  Dispose in an approved waste facility according to Federal, State & Local regulations.

## SECTION 10 – STORAGE AND HANDLING INFORMATION

Precautions to be Taken in Handling and Storage: No special requirements if product is handled according to good manufacturing practices.  Avoid contact with eyes, skin and clothing.  Keep container closed when not in use.  Wash thoroughly after handling.         D.O.T. Classification:  Not regulated.

The Health hazards given on this Material Safety Data Sheet Apply to this product in its concentrated form (as supplied)  and may differ significantly at use dilution.  The signs and symptoms of overexposure apply only to negligence in handling or misuse of the concentrated product and not to the routine exposure of the diluted product under conditions of ordinary use.

HMIS HAZARD RATING

| Health | 2 |
| --- | --- |
| Flammability | 0 |
| Reactivity | 0 |
| Personal Protection | X |

0 = Least, 1 = Slight, 2 = Moderate, 3 = High, 4 = Extreme