MATERIAL SAFETY DATA SHEET

Date Prepared:  7-1-11

SECTION 1 – PRODUCT INFORMATION

Distributor:    Swiss Clean, Inc.

Address:  3650 – 44th St. SE, Kentwood, MI  49512    	Emergency Phone:  1-800-424-9300

Trade Name:  Phoenix Renew				Product Type: Floor Maintenance Cleaner/Restorer.

## SECTION 2 – INGREDIENTS

| Chemical Name/Common Name | CAS No. | % (Optional) | TLV (Source) | TITLE III, SECT. 313 |
| --- | --- | --- | --- | --- |
| Cocamide-DEA | 68603-42-9 | 1 – 10% | None Established | No |
| D-Limonene | 5989-27-5 | 1 – 10% | None Established | No |
| Water | 7732-18-5 |  | None Established | No |

## SECTION 3 – PHYSICAL DATA

Boiling Point (°F): 212º		 Specific Gravity (H2O=1.0): 1.0 + 0.005 	pH:  9.9 + 0.5

Vapor Pressure (mm Hg): Not Determined		 	Vapor Density (Air=1):  Not Determined

Solubility in water:  Complete 			 	Evaporation Rate (vs. H2O): About the Same

Appearance and Odor: Pale straw colored liquid with citrus odor.

Slip Resistance:  This product meets or exceeds standards for slip resistance (ASTM D 2047-75), but manufacturer/distributor

is not responsible for slips or falls when product is used on quarry tile, ceramic, brick, marble, stone or other hard or irregular floors.

# SECTION 4 – FIRE AND EXPLOSION HAZARD DATA

Flash Point (T.C.C.):  None to Boiling		Flammable Limits:  N/A

Extinguishing Method:  Product is non-combustible.  Water Spray (Fog), Dry Chemical; CO2: or Foam may be used where

product is stored.

Special Firefighting Procedures:  Do not enter confined fire-spaces without protective clothing and self-contained air supply.

Unusual Fire and Explosion Hazards:  None

## SECTION 5 – REACTIVITY DATA

Stability:   Stable					Incompatibility:  Strong acids/oxidizers

Hazardous Decomposition Products:  Burning may product oxides of carbon and other substances.

## SECTION 6 – HEALTH HAZARD DATA

Primary Routes of Exposure:   Eyes, Skin, Oral

Signs and Symptoms of Overexposure (Acute):  Direct contact with eyes may cause mild to moderate irritation with local redness & swelling of conjunctiva.  Prolonged or repeated skin contact may cause irritation.  Ingestion may cause irritation of mouth, throat & stomach with possible nausea, vomiting and C.N.S. depression (dizziness, fatigue, headache).

Signs and Symptoms of Overexposure (Chronic):  Repeated or extended exposure may cause an allergic skin reaction.

Medical Conditions Aggravated by Overexposure: None currently known.

## SECTION 7 – EMERGENCY AND FIRST AID PROCEDURES

Eyes:  Flush eyes with water for at least 15 minutes and call a physician immediately.

Skin:   Wash affected area with large amounts of soap and water.  If irritation persists, call a physician.

Inhalation:    Not an expected route of exposure.  If vapors or mist cause irritation, remove subject to fresh air.

Ingestion:    Do NOT induce vomiting.  Dilute by drinking water.  Call a physician immediately.

## SECTION 8 – SPECIAL PROTECTION INFORMATION

Respiratory Protection:  Not required under normal use conditions with good general ventilation.

Ventilation Requirements:  Normal  	Eye Protection:	Safety Glasses/Goggles	Protective Gloves:  Rubber/PVC

Other Protective Clothing:  Protective clothing appropriate to minimize contact during specific operations.

## SECTION 9 – SPILL OR LEAK PROCEDURES

Steps to be Taken if Released or Spilled:  Spills can create extreme slip hazard.  Stop leak if you can do it without risk.  Restrict

entry to spill area and dike large spills to prevent run off.  Mop, pump or take up with an inert absorbent and reclaim for reuse,

recycle or proper disposal.

Waste Disposal Methods:  Dispose of in an approved waste facility according to Federal, State and Local regulations.

## SECTION 10 – STORAGE AND HANDLING INFORMATION

Precautions to be Taken in Handling and Storage:  DO NOT FREEZE.  Do not use on surfaces colder than 45ºF.  Floors can be slippery when wet.  Clean up spills promptly.  Failure to follow these precautions could lead to product failure or slippery conditions.

D.O.T.  Classification:   Not regulated

The Health hazards given on this Material Safety Data Sheet Apply to this product in it’s concentrated form (as supplied)  and may differ significantly at use dilution.  The signs and symptoms of overexposure apply only to negligence in handling or misuse of the concentrated product and not to the routine exposure of the diluted product under conditions of ordinary use.

HMIS HAZARD RATING

| HEALTH | 1 |
| --- | --- |
| FLAMABILITY | 0 |
| REACTIVITY | 0 |
| PERSONAL PROTECTION | B |

0=Least, 1=Slight, 2= Moderate, 3=High, 4=Extreme